S32K144 EVB User Manual

- Get to know S32K144-EVB.

- S32K144 EVB Features:

  - Supports S32K144 100LQFP.

  - External Crystal 8MHz.

  - Easy access to the MCU I/O header pins for prototyping.

  - On-chip connectivity for CAN, LIN, UART, I2C.

  - CAN TJA1051T/3 and LIN TJA1027.

  - Potentiometer for precise voltage and analog measurement.

  - RGB LED.

  - Two push-button switches (BTN0 and BTN1).

  - DS1307 and AT24C32 for I2C communication practice.

  - External flash W25Q16 for SPI communication practice.

  - LED 7-Segment with 74HC595 for SPI communication practice.

  - External 7-12V via jack DC 5.5x2.1

- Jumper Settings

| Jumper | Configuration | Description |
| --- | --- | --- |
| JP2 | LIN-M: For Lin Master LIN-S: For Lin Slave | Select LIN communication role Master or Slave |
| JP3 | 1-2: LIN SLP driver by GPIO Pin. 2-3: LIN SLP Connect to 5V-DC | Select LIN SLP connection |
| JP4 | Short: Enable CAN 120R resistor | Enable/Disable CAN 120R resistor |

- GPIO Mapping

| Component | S32K144 |
| --- | --- |
| LED RGB RED | PTD15 (FTM0 CH0) |
| LED RGB BLUE | PTD0(FTM0 CH2) |
| LED RGB GREEN | PTD16(FTM0 CH1) |
| Potentiometer | PTA6 (ADC0_SE2) |
| BTN0 | PTC12 |
| BTN1 | PTC13 |
| CAN TX | PTE5(CAN0_TX) |
| CAN RX | PTE4 (CAN0_RX) |
| LIN TX | PTD7(LPUART2_TX) |
| LIN RX | PTD6 (LPUART2_RX) |
| DS1307/AT24C32 I2C SDA | PTA2 (LPI2C0_SDA) |
| DS1307/AT24C32 I2C SCL | PTA3 (LPI2C0_SCL) |
| 74HC595 SER | PTC1 (LSPI2_SOUT) |
| 74HC595 SRCLK | PTC15 (LSPI2_SCK) |
| 74HC595 RCLK | PTC14 (LSPI2_PCS0) |
| W25Q16 DI | PTB4 (LSPI0_SOUT) |
| W25Q16 DO | PTB3 (LSPI0_SIN) |
| W25Q16 SCK | PTB2 (LSPI0_SCK) |
| W25Q16 CS | PTB0 (LSPI0_PCS0) |
| Jump J6 TXD Pin | PTC7 (LPUART1_TXD) |
| Jump J6 RXD Pin | PTC6 (LPUART1_RXD) |

- Component Schematic.

- CAN Communication.

- LIN Communication.

- LED RGB.

- User Button.

- Potentiometer.

- DS1307/AT24C32.

- External Flash W25Q16.

- LED 7-Segment 74HC595.